Öz:Malatya güneyinde geniş yüzlekleri bulunan ve dolomitik mermer ve mermerlerden oluşan Koltik kireçtaşı yaklaşık 1000 metre kalınlığa sahiptir. Çalışma alanının temelini Yüksekova karmaşığı (Üst Kretase) oluşturur. Koltik kireçtaşı Yüksekova karmaşığı üzerine tektonik (faylı) bir dokanakla gelir. Daha genç birimler Yüksekova karmaşığı ve Koltik kireçtaşını uyumsuz olarak üzerler. Kireçtaşının jeomekanik özelikleri arazi ve laboratuvar çalışmaları ile belirlenmiştir. Arazi çalışmalarında, kireçtaşının orta-sık çatlaklı ve çatlakların genellikle dolgulu oldukları gözlenmiştir. Laboratuvarda kaya bloklarından elde edilen küp örnekler üzerinde TSE ve ASTM standartlarına göre deneyler yapılmıştır.Çalışılan kireçtaşının katman kalınlığı, birim hacim ağırlığı, su emme özelliği, tek eksenli basınç dayanımı, asit etkilerine dayanımı ve don kaybı TSE standartlarına , tek eksenli basınç dayanımı, asit etkilerine dayanımı ASTM standartlarına göre uygun sınırlar içindedir. Deney sonuçlarına göre, Koltik kireçtaşının yapı ve kaplama taşı olarak kullanılabilir olduğu belirlenmiştir.

Başlık (İngilizce):Utilization as building and cladding stone of Permo-Carboniferous age Koltik limestone (Malatya southern)

Öz (İngilizce):Koltik limestone has approximately 1000 m thick, which is wide appearences in the southern part of Malatya. The basement rock of study area is Y&uuml;ksekova complex (Upper Cretaceous). Koltik limestone (Permo-Carboniferous) which is mainly composed of dolomitic marbles and marbles, was placed on the Y&uuml;ksekova complex with a tectonic (fault) contact. Upper Cretaceous and Tertiary sediments overlain to the Y&uuml;ksekova complex and Koltik limestone as unconformably.The geomechanics features of the Koltik limestone were determined with area and laboratory studies. Limestone is middle to often cracked. Cracks were generally filled. In the laboratory, on the cubical formed samples from the rock blocks were done experiments as to TSE and ASTM standarts. According to TSE standarts bed thickness, unit weight, specific gravity, water absorbtion, uniaxial compressive strength, resistivity to acidity and frost action values were found to be suitable for marble industries. According to ASTM standarts uniaxial compressive strength and resistivity to acidity are suitable for marble industries.As to the results of the experiments, Koltik limestone may be used for building and cladding stone.